Contact Info:

Woody Underwood

Dr. Richard Jones' Lab

University of Connecticut, Department of Physics

2152 Hillside Road

Storrs, CT 06269

Cell phone: (860) 306-9234

Board Specs:

- 6940x1650 mil

- 6 layer

- 0.005” trace/space

- Permanent white epoxy silkscreen (top and bottom)

- *Black* solder mask on top and bottom

- Finished thickness: ~96 mil (approximately… we need it thicker than the standard 62 mil, so as close as you can get to 96 mil)

- Lead requirements: doesn’t matter (whatever is cheapest, we have no ROHS requirements)

- Material type: *black* (opaque) FR-4, 1 oz copper

- Electrical testing: Yes

- SMT pads

  - Top: 2

  - Bottom: 0

- PTH Drills

  - 27.559 mil, 61

  - 35.433 mil, 20

  - 39.370 mil, 144

  - 47.244 mil, 6

  - 110.236 mil, 4

  - 138 mil, 6

- Front to back registration, less than 5 mil

- Warp and twist 7 mil per inch or better

- Impedance controlled: 7 mil traces on top and bottom layers should have approximately 50 Ohm impedance against plane 1 and plane 4 respectively

Here is the layer stackup (by gerber file extensions):

GTO = Top silkscreen

GTP = Top solder paste

GTS = Top solder mask

GTL = Top layer

GP1 = Internal plane 1

GP2 = Internal plane 2

GP3 = Internal plane 3

GP4 = Internal plane 4

GBL = Bottom layer

GBS = Bottom solder mask

GBP = Bottom solder paste

GBO = Bottom silkscreen

Also attached are drill files for the board: DRL, DRR, TXT

If I have omitted anything, please let me know.